Release Notes for BUILD Calculator Version 2.0.0:

- New mixed-fuel baseline to align with the 2022 Energy Code prescriptive standards as specified by the 2022 Single-Family Residential and Multifamily ACM Reference Manuals. The mixed-fuel baseline changes include:

  - Single-Family

    - Space Heating — Heat pump in climate zones (CZ) 3, 4, 13, and 14. Gas furnace in CZ 1, 2, 5-12, 15, and 16.

    - Water Heating — NEEA T3 heat pump in CZ 1, 2, 5-12, 15, and 16. Gas tankless in CZ 3, 4, 13, and 14.

  - Low-Rise Multifamily (Three stories or fewer)

    - Space Heating — Heat pump in CZ 1-15. Gas furnace in CZ 16.

    - Unitary Water Heating — Gas tankless in all CZ.

    - Central Water Heating — Gas central in all CZ.

  - Mid- and High-Rise Multifamily (Four stories or more)

    - Space Heating — Heat pump in CZ 2-15. Dual-fuel heat pump in CZ 1, 16.

    - Unitary Water Heating — Gas tankless in all CZ.

    - Central Water Heating — Gas central in all CZ.

    - Solar Photovoltaic (PV) — Now required with certain conditions and exceptions based on available solar access roof area.

    - Battery — Now required with certain conditions and exceptions.

- Updated heating ventilation air conditioning (HVAC) System for mid- and high-rise multifamily calculators to include additional system types.

- Updated domestic hot water (DHW) system for mid- and high-rise multifamily calculators to include additional system types.

- Added laundry for mid- and high-rise multifamily calculators.

- Added envelope measures for mid- and high-rise multifamily calculators.

- Removed battery in Step 1 for mid- and high-rise multifamily calculators.

- Removed DHW Location for all calculators.

- Removed drain water heat recover (DWHR) for all calculators.

- Reduced Incremental PV incentive from $3.00 per watt to $1.30 for mid- and high-rise multifamily calculators.