Station Checklist

Station Name: _____________________________________________________________________

Station Location (Street Address, City, State, Zip):

_____________________________________________________________________

Recipient Name: _____________________________________________________________________

Agreement Number: _____________________________________________________________________

Technology Type:

_____________________________________________________________________

By completing and signing this Station Checklist, the Recipient is self-certifying that the station listed above meets and will adhere to each of the Minimum Technical Requirements for Stations of GFO-25-XXX.

Should this station come out of compliance with the Checklist, or should the design change, the Recipient shall submit to the CEC a new completed, signed, and dated Station Checklist.

| Check | Hydrogen Refueling Station Requirements |
| --- | --- |
|  | Each dispenser has passed a hydrogen quality test and dispenses hydrogen that complies with California Code of Regulations (CCR) Title 4 Business Regulations, Division 9 Chapter 6 Automotive Products Specifications, Article 8, Hydrogen Fuel Sections 4180 and 4181, which adopts SAE International J2719 (fuel quality). |
|  | (If applicable) Each dispenser underwent type evaluation testing and has a Temporary Use Permit or Certificate of Approval issued through California Type Evaluation Program, or a Certificate of Conformance issued by the National Type Evaluation Program. |
|  | (If applicable) Each dispenser has successfully passed initial verification of accuracy class tests to receive the county weights and measures seal approving the device for retail use. |
|  | The station conforms to National Fire Protection Association (NFPA) 2. |
|  | Optional: Does the station include infrastructure to support fueling of vehicles with compressed hydrogen storage systems (CHSS) that exceed 10 kilograms? Yes No If Yes, check the box above that the infrastructure conforms to a defined fueling protocol that the Recipient has described to the California Energy Commission (CEC). If No, leave blank. |
|  | The station conforms to the most recent version of ANSI/CSA HGV 4.9 (hydrogen refueling stations). |
|  | The station conforms to the most recent version of SAE International J2799 (station communications), verified through the most recent version of CSA HGV 4.3. or an equivalently accepted industry standard. |
|  | Each station nozzle conforms to the most recent version of SAE J2600 (nozzles) or International Organization for Standardization (ISO) 17268 (nozzles). Note: Fast fills, (up to 7.2kg/min) require a different nozzle with a different standard (ISO 27268:2012) and are permitted for heavy duty vehicles only. |
|  | The station conforms to one or more of the following fueling protocols or an equivalently accepted industry standard: - J2601 – 1 Category D (greater than 10 kg tank sizes) - J2601 – 2 HD fueling - J2601 – 4 Ambient Temperature refueling - J2601 – 5 MC Method for HD fueling - JPEC-S 0003 Japanese Bus fueling protocol |
|  | The station conforms with the American National Standards Institute (ANSI) Standards - Hydrogen Gas Vehicle (HGV) 2-2021 - HGV 4.1 - G 095A - HPRD 1:21 - HGV 3.1 - CGA S1.1 |
|  | The station conforms with the ISO Standards: - 19880-3 - 19880-4 - 19880-5 - 19880-6 |
|  | The station conforms with the California Building Codes: - California Building Code, Part 2, Title 24 - California Electrical Code, Part 3, Title 24 - California Energy Code, Part 6, Title 24 - California Fire Code, Part 9, Title 2 |
|  | The station conforms with the California Department of Food and Agriculture, Division of Measurement Standards (DMS) Testing Standards: - Handbook 44 Section 3.34 - Handbook 44 Section 3.39 - NIST Handbook 130 |
|  | The station sells fuel to the public through a point of sale (POS) that accepts, reads, and processes the magnetic stripe on commercially available credit cards, debit cards, fueling cards, and gift cards, if applicable. The POS also reads EMV™ chips embedded in the cards and performs financial payment transactions. |
|  | Optional: Does the station POS system wirelessly transmit, receive, and process near-field communications (NFC) to process the signals from contactless cards or mobile devices, i.e., “smart phones,” or accept payment through a mobile application? Yes No If Yes, check the box above. If No, leave blank. |
|  | The station’s refueling components are installed and the station has a dedicated hydrogen fuel supply and delivery agreement from a hydrogen production plant (on or off-site), with available capacity, and a second supply arrangement as backup. |
|  | The station has an energized utility connection and source of system power. |
|  | The station has lighting for the dispenser(s) and the station area to provide a well-lit area that is safe, convenient, and accessible for station users, if applicable. |
|  | The station has onsite signage that explains the method of sale requirements, if applicable. |
|  | If approved by the respective authority: Highway and trailblazer signage is installed. Has Caltrans approved state highway signage? Yes No Has the AHJ approved trailblazer signage on local roads? Yes No |
|  | If applicable, each dispenser is connected and reporting all required information to the Hydrogen Fuel Cell Partnership Station Operational Status System (SOSS). |
|  | The station has received all required state, local, county, and city permits to build and operate. |
|  | The station has a guard or cover installed over the station emergency shutdown system switch(es). |
|  | If Public Access, the station is accessible to the public: • No obstructions or obstacles exist to preclude vehicle operators from entering the station premises. • The user of the station is not required to obtain or to use access cards or personal identification (PIN) codes for the station to dispense fuel. • No formal or registered station training is required for individuals to use the hydrogen refueling station. |

| Certification: I hereby authorize the CEC to make any inquiries necessary to verify the information presented in this checklist. I hereby certify to the best of my knowledge that the station has been constructed and equipment has been installed consistent with the CEC agreement and the information contained in this checklist is correct and complete. I certify that the station will remain operational for a minimum of six years and will meet and adhere to these Station Requirements during station operation. |  |  |  |
| --- | --- | --- | --- |
| Name and Signature of Authorized Representative |  | Date: |  |