# BUILD Custom Path Tool Instructions

Disclaimer: BUILD is administered by the California Energy Commission in collaboration with the California Public Utilities Commission. It is authorized by Senate Bill 1477 (2018, Stern) and funded by the four California gas corporations apportioned according to each gas corporation’s percentage share of allocated Cap-and-Trade Program allowances. Reservations are approved on a first come, first served basis, and regional funding availability is based on the utilities’ contribution to the program.

# Introduction

The California Energy Commission (CEC) administers the Building Initiative for Low-Emissions Development (BUILD) Program, a residential building decarbonization program providing incentives and technical assistance to support the adoption of advanced building design and near-zero-emission technologies in new all-electric low-income homes. BUILD incentives are based on a project’s anticipated modeled greenhouse gas (GHG) emission reduction as compared to the mixed-fuel 2019 Energy Code prescriptive standards. Projects receiving BUILD incentives cannot result in higher utility bills for building occupants.

BUILD staff developed the BUILD Custom Path Tool (Tool) for applicant’s use to calculate the building’s incentive amounts as described in the BUILD Guidelines. These instructions show how to use the Custom Path Tool to calculate incentives for newly constructed buildings. Staff may update these instructions should any changes be made to the Custom Path Tool. Such changes will be outlined on the BUILD Incentives page accessible on the BUILD webpage by clicking “More Information” under Incentives.

For questions concerning any of the instructional content, please contact BUILD staff at BUILD@energy.ca.gov.

Contents

Introduction	1

1. Getting Started	4

2. Creating Additional Files for Low-rise Residential Buildings	6

2.1 File #1 (For all building types)	8

2.2 File #2 (For all building types)	10

2.3 File #3 (Only for multifamily buildings with central laundry)	11

2.4 File #4 (Only for multifamily buildings with central laundry)	12

3. Creating Additional Files for Mid- and High-rise Residential Buildings	14

3.1 File #1 (For all Mid- and High-rise multifamily buildings)	16

3.2 File #2 (For all Mid- and High-rise multifamily buildings)	17

3.3 File #3 (For Mid- and High-rise multifamily buildings with central laundry)	19

3.4 File #4 (For Mid- and High-rise multifamily buildings with central laundry)	20

4. Launching the Tool	21

Step 1: Complete the project information and building features	21

Step 1.1: Project Information	21

Step 1.2: Laundry	22

Step 1.3: Domestic Hot Water (DHW)	22

Step 1.4: Photovoltaic (PV) System	23

Step 1.5: Compliance	24

Step 1a (optional): Include any kickers to qualify for additional incentives	24

5. Calculations	26

Step 2: Press the Calculate Button	26

Step 3: Confirm project meets the program’s modeled resident cost savings requirement	26

Step 4: Options to meet modeled resident utility cost savings	27

Step 5: Cost Savings Difference	29

6. Review and Save Results	31

Step 6: Review Results and Estimated Incentive Amount	31

Step 6.1: Percent Better Than Title 24 and GHG Results	31

Step 6.2: Incentive Calculation	31

Step 6.3: Save a Copy	32

# 1. Getting Started

To use the newest version of the BUILD Custom Path tool, download the zip file from the BUILD Incentive webpage and unzip to a new folder. The zip file contains these instructions and the Tool, which is an Excel spreadsheet embedded with macros and formulas. The spreadsheet is relatively large with many automatic calculations so it may take several minutes for the file to completely load. The user may encounter several warnings when loading the spreadsheet due to first-time use and macros it contains (Figure 1).

Figure 1

Click the “Enable” buttons to continue.

Figure 2

The Tool opens on the screen similar to the one displayed above (Figure 2). With the release of Custom Path Tool v1.3, only this version is needed to calculate the incentive values for a Single-Family, Low-Rise Multifamily, and Mid- and High-rise Multifamily building. Users enter information on the left side in sequential order (top-to-bottom, left-to-right) as it pertains to the building. Once completed, and the additional files are created (as explained later) and located in the appropriate folder, the user clicks the “Calculate” button at Step 2 which initiates the macros to search the additional files. There may be additional user information needed at Step 3 (as explained later) and once complete, the results and incentives will be displayed in Step 4.

The spreadsheet can be renamed and saved or click the “Print to PDF” button for archival purposes.

For low-rise residential buildings (single family and multifamily buildings with 3 stories or less), go to Section 2. For mid- and high-rise residential buildings (multifamily buildings with 4+ stories), go to Section 3.

# 2. Creating Additional Files for Low-rise Residential Buildings

The Custom Path Tool for residential single-family and multifamily buildings 3-stories or less relies on several special output files created by California’s Building Energy Code Compliance Software – Residential (CBECC-Res). The Tool, version 1.3, is calibrated to use output files created by CBECC-Res 2019.2.0 (SP1 & SP2).  The tool will be updated as newer versions of CBECC are released.

Pursuant to the BUILD Guidelines, a qualified residential building includes the dwelling units and associated residential common areas (for example - hallways, laundry rooms, leasing office, and other spaces intended for residents only, including attached unconditioned garages). For mixed-use buildings with residential units, only the dwelling units and residential common areas qualify for incentives. The mixed-use common spaces are intended only for the residents and not the public.

To get started, a CBECC input file (known as the energy model) developed by a Certified Energy Analyst (CEA), using currently approved software for the qualifying building spaces is required.  The energy model can only contain the dwelling units and residential common areas described above, which should be identical to the energy model that would be used for the building permit. For a mixed-use energy model, remove the non-qualifying spaces.

Set a CEBECC “Program INI” setting to create the hourly energy output files (Figure 3). It can be set from the CBECC user interface by clicking on the menu Tools => Program and Analysis Options and the Program INI Settings window should appear. Check the “Export Hourly Results for ALL Runs” checkbox then click the “Done” button.

Figure 3

With the release of Custom Path Tool v1.3, the previous FILE3 0 PV file is no longer needed and the Custom Path Tool will look for two or four output files from two or four different energy models for multifamily buildings. Only two files are needed if all the dwelling units have in-unit laundry machines, and four files are needed if the building has central laundry. Single-family buildings will only need two energy models since they would have in-unit laundry. The files must have the text ‘FILE1’, ‘FILE2’, etc., added to the beginning of the file name and be located in the same folder. It is recommended for each file to rename and save the energy models with the specific name pattern before running the energy model. The four file concept is as follows:

FILE1 – Proposed model with qualifying spaces

FILE2 – Proposed model converted to mixed fuel

FILE3 – Mixed fuel model without laundry (for multifamily with central laundry)

FILE4 – Proposed model without laundry (for multifamily with central laundry)

Example file naming of the energy models:

Figure 4

After running the energy models, CBECC will produce hourly-energy-use files that shows the energy use of various components for each hour of a year (8,760 hours). The Tool reads these files to calculate the Modeled Resident Utility Costs, as described in the BUILD Guidelines.

Example file naming of the hourly-energy-use files:

Figure 5

## 2.1 File #1 (For all building types)

The first file, FILE1, is the energy model of the proposed building with only the qualifying spaces included and must be all-electric (Figure 6). Ensure the fuel type for all appliances is set to “Electricity”.  The laundry appliances should be modeled in all dwelling units as being in the individual dwelling unit or in a common central laundry room based on the project’s design. If the project has a central laundry room that is detached from the residential building, model it as ‘central’ in all the dwelling units to account for the associated GHG emissions.

Figure 6

Next, run the FILE1 energy model using CBECC to completion. It will create numerous files in the folder where the energy model file is located. Locate the .CSV file named “FILE1...Prop - HourlyResults.csv” and copy it to an empty folder.

For example:

Figure 7

For a building run using the Multiple Orientation Analysis without a specific building front-facing orientation:

Figure 8

The correct “...Prop - HourlyResults.csv” file to use would be based on the orientation with the lowest Energy Design Rating (EDR) Compliance Margins.  For example, using the compliance margins below (Figure 9), the West Facing orientation has the lowest EDR Compliance Margins.  Therefore, the “...Prop - HourlyResults.csv” file would have a ‘W’ for West in the name, such as “...Prop-W HourlyResults.csv”.  North Facing would have an ‘N’, East Facing would have an ‘E’, and South Facing would have an ‘S’.  This same methodology should be used for FILE 3 and FILE5, explained subsequently.

Figure 9

## 2.2 File #2 (For all building types)

The BUILD Guidelines describe how the Modeled Resident Utility Costs are calculated by comparing the modeled resident utility cost of the proposed all-electric building to that of a code-compliant mixed-fuel building. CBECC has the ability to create the hourly-energy-use file for the code-compliant mixed-fuel building automatically (mixed-fuel Standard Design model), but the proposed model needs to be changed to a mixed-fuel version to cause CBECC to create the proper standard design file.

To create FILE2, copy and rename the FILE1 energy model to FILE2. Then modify it to become a mixed-fuel building with gas space heating, gas water heating, gas cooking, and gas laundry appliances. It is important to keep the systems that are central systems (such as central laundry room, central water heating) consistent when changing them from electricity to gas. The capacities and efficiencies of the gas-fired equipment in FILE2 are not important because they are just needed to be in the model to cause CBECC to create the mixed-fuel Standard Design file.

Run the FILE2 energy model using CBECC to completion and it does not matter if it complies or not. There must be Standard Design therms shown for Space Heating, Water Heating, and Appliances & Cooking when viewing the Energy Use Details tab (Figure 10).

Figure 10

Locate the .CSV file named “FILE2...Std - HourlyResults.csv” and copy it to the same folder with the other files. It is important that the file name has “Std” and not “Prop”.

For example:

Figure 11

## 2.3 File #3 (Only for multifamily buildings with central laundry)

To create FILE3, start with the FILE2 energy model (copy and rename as FILE3) and modify it to remove laundry appliances from all dwelling units by setting the Clothes Washer and Clothes Dryer to ‘-none-‘ (Figure 12).  This file is needed so the Custom Path Tool can calculate the energy use of the laundry appliances and their associated GHG emissions when compared to FILE2.

Figure 12

Run the FILE3 energy model using CBECC to completion and it does not matter if it complies or not. Locate the .CSV file named “FILE3...Std - HourlyResults.csv” and copy it to the same folder with the other files.  It is important that the file name has “Std” and not “Prop”.

For example:

Figure 13

## 2.4 File #4 (Only for multifamily buildings with central laundry)

To create FILE4, start with the FILE1 energy model (copy and rename as FILE4) and modify it to remove the laundry appliances from all dwelling units by setting the Clothes Washer and Clothes Dryer to ‘-none-‘ (Figure 14). This file is needed so the Custom Path Tool can calculate the energy use of the laundry appliances and their associated GHG emissions when compared to FILE1.

Figure 14

Run the FILE4 energy model using CBECC to completion and it does not matter if it complies or not. Locate the .CSV file named “FILE4...Prop - HourlyResults.csv” and copy it to the same folder with the other files.

For example:

Figure 15

# 3. Creating Additional Files for Mid- and High-rise Residential Buildings

The Custom Path Tool for residential multifamily buildings 4-stories and higher relies on several special output files created by California’s Building Energy Code Compliance Software – Commercial (CBECC-Com). The Tool, version 1.3, is calibrated to use output files created by CBECC-Com 2019.2.0.SP1.  The Tool will be updated as newer versions of CBECC are released.

Pursuant to the BUILD Guidelines, a qualified residential building includes the dwelling units and its associated residential common areas (for example - hallways, laundry rooms, leasing office, and other spaces intended only for residents, including attached unconditioned garages). For mixed-use buildings with residential units, only the dwelling units and residential common areas qualify for incentives. The common spaces are intended only for the residents and not the public.

To get started, a Certified Energy Analyst (CEA) creates a CBECC input file (known as the energy model) using currently approved software for the qualifying spaces of the building. The energy model can only contain the dwelling units and residential common areas described above and must be identical to the energy model used for the building permit. For a mixed-use energy model, remove the non-qualifying spaces. The qualifying energy model must show compliance with the Energy Code when run with CBECC using the ‘compliance’ speed simulation option.

A “Program INI” setting will need to be set in CBECC in order to create the hourly energy output files. It can be set from the CBECC user interface by clicking on the menu Tools => Program and Analysis Options and the Program INI Settings window should appear. Check the “Export All Hourly Simulation Results” checkbox then click the “Done” button (Figure 16).

Figure 16

With the release of Custom Path Tool v1.3, the previous FILE3 0 PV file is no longer needed, and the Custom Path Tool will look for two or four output files from two or four different energy models for multifamily buildings. Only two files are needed if all the dwelling units have in-unit laundry machines, and four files are needed if the building has central laundry. Single-family buildings require two energy models since they would have in-unit laundry. The filenames must begin with ‘FILE1’, ‘FILE2’, etc., added to the beginning of the file name and be located in the same folder. It is recommended for each file to rename and save the energy models with the specific name pattern before running the energy model. The four-file concept is as follows:

FILE1 – Proposed model with qualifying spaces

FILE2 – Proposed model converted to mixed fuel

FILE3 – Mixed fuel model without laundry (for multifamily with central laundry)

FILE4 – Proposed model without laundry (for multifamily with central laundry)

Example file naming of the energy models:

Figure 17

After running the energy models, CBECC produces hourly-energy-use files that show the energy use of various components for each hour of a year (8,760 hours). The Tool reads these files to calculate the Modeled Resident Utility Costs, as described in the BUILD Guidelines.

Recommended file naming of the hourly-energy-use files are as follows:

Figure 18

## 3.1 File #1 (For all Mid- and High-rise multifamily buildings)

The first file, FILE1, is the energy model of the proposed building with only the qualifying spaces included and must be all-electric. Ensure all the appliances are set to “Electric”. To account for the laundry appliance energy and associated GHG emissions, check the “Dwellings Have Clothes Washer” and “Dwellings Have Clothes Dryer” checkboxes in all dwelling units whether they have in-unit laundry or a common central laundry room (Figure 19). Even if the project has a central laundry room that is attached or detached from the residential building, check the checkboxes as described for all dwelling units.

Figure 19

Next, run the FILE1 energy model using CBECC to completion. It will create numerous files in the folder where the energy model file is located and will create a subfolder with a name similar to the file name (Figure 20).

Figure 20

Go to the subfolder and locate the .CSV file named “FILE1…- ap - HourlyResults.csv” and copy it to the same folder with the other files.

For example:

Figure 21

## 3.2 File #2 (For all Mid- and High-rise multifamily buildings)

The BUILD Guidelines describe how the Modeled Resident Utility Costs are calculated by comparing the modeled resident utility cost of the proposed all-electric building to that of a code-compliant mixed-fuel building. CBECC can create the hourly-energy-use file for the code-compliant mixed-fuel building automatically (mixed-fuel Standard Design model), but the proposed model needs to be changed to mixed-fuel to cause CBECC to create the proper file.

To create FILE2, copy and rename the FILE1 energy model to FILE2. Then modify it to become a mixed-fuel building with gas space heating, gas water heating, gas cooking, and gas laundry appliances. It is important to keep the systems that are central systems (such as central laundry room, central water heating) consistent when changing them from electricity to gas. The capacities and efficiencies of the gas-fired equipment in FILE2 are not important because they are just needed to be in the model to cause CBECC to create the mixed-fuel Standard Design file.

Run the FILE2 energy model using CBECC to completion, it does not matter if it complies or not. There must be Standard Design therms shown for Space Heating, Water Heating, and Process when viewing the Energy Use Summary tab (Figure 22).

Figure 22

Go to the FILE2 subfolder and locate the .CSV file named “FILE2…- ab - HourlyResults.csv” and copy it to the same folder with the other files. It is important that the file name has “ab” and not “ap”.  “ab” represents the Standard Design model results and “ap” represents the proposed model results.

For example:

Figure 23

## 3.3 File #3 (For Mid- and High-rise multifamily buildings with central laundry)

To create FILE3, start with the FILE2 energy model (copy and rename as FILE3) and modify it to remove all laundry appliances by unchecking the Dwellings Have Clothes Washer and Dryer checkboxes (Figure 24). This file is required for the Custom Path Tool to calculate the energy use of the laundry appliances and their associated GHG emissions when compared to FILE2.

Figure 24

Run the FILE3 energy model using CBECC to completion, it does not matter if it complies or not. Go to the FILE3 subfolder and locate the .CSV file named “FILE3…- ab - HourlyResults.csv” and copy it to the same folder with the other files.  It is important that the file name has “ab” and not “ap”.  “ab” represents the Standard Design model results and “ap” represents the proposed model results.

For example:

Figure 25

## 3.4 File #4 (For Mid- and High-rise multifamily buildings with central laundry)

To create FILE4, start with the FILE1 energy model (copy and rename as FILE4) and modify it to remove all laundry appliances by unchecking the Dwellings Have Clothes Washer and Dryer checkboxes (Figure 26). This file is needed so the Custom Path Tool can calculate the energy use of the laundry appliances and their associated GHG emissions when compared to FILE1.

Figure 26

Run the FILE4 energy model using CBECC to completion and it does not matter if it complies or not. Go to the FILE4 subfolder and locate the.CSV file named “FILE4…- ap - HourlyResults.csv” and copy it to the same folder with the other files.

For example:

Figure 27

# 4. Launching the Tool

Once the additional files are placed in the proper folder, launch the Tool. It should load and display the user interface sheet. Fill out the information requested as follows:

## Step 1: Complete the project information and building features

Complete the applicant input boxes by entering in the project information or using the drop-down options.

### Step 1.1: Project Information

Figure 28

Project Name: Enter the project name. If this information is not available yet, enter “N/A”.

Project Address: Enter the project address. If this information is not available yet, enter “N/A”.

Building Name: Enter the building name or names if representing multiple buildings (e.g., “Building 1” or “Buildings 1-5”). If this information is not available or not applicable, enter “N/A”.

Project Zip Code: Enter the project zip code.

For buildings that are master-metered or where all the tenants will not pay electric utility bills, check the box and the Tool will skip Step 3, described below.

Figure 29

Select Climate Zone: Use the drop-down menu to select the climate zone. An applicant may use the CEC’s climate zone tool to determine the climate zone.

Select Gas Utility: Use the drop-down menu to select the gas utility. If the project’s utility is not listed in the drop-down menu, choose the closest investor-owned utility available in the drop-down.

Select Electric Utility: Use the drop-down menu to select the electric utility. If the project’s utility is not listed in the drop-down menu, choose the closest investor-owned utility available in the drop-down.

Total Conditioned Residential Floor Area: Enter the total square footage for the conditioned residential floor area. This includes common areas such as hallways and laundry rooms.

No. of Stories: Enter the total number of stories above grade.

Total No. of Units: Enter the total number of dwelling units for the entire project. This box does not apply to the single-family tool.

Total No. of Bedrooms: Enter the total number of bedrooms for the entire project.

### Step 1.2: Laundry

Figure 30

Central Laundry: Select whether the building has central laundry for tenants. Buildings with in-unit laundry machines and single family should select “No”.

### Step 1.3: Domestic Hot Water (DHW)

Figure 31

Central Water Heating: Select whether the building has a central water heating system. Buildings with unitary domestic hot water systems and single family should select “No”.

### Step 1.4: Photovoltaic (PV) System

Figure 32

Minimum PV size to achieve compliance: Enter the minimal PV system size needed to achieve compliance. Some energy models include more PV modeled to match the building’s PV design than is required by the Energy Code. Since BUILD only incents the incremental PV above what is required for modeled resident utility cost compliance, only that minimal amount is needed for this input. The PV system modeled in FILE1 should be the same system size from the Certificate of Compliance used to apply for the building permit. For Energy Code compliant mid- and high-rise multifamily buildings that do not require PV, enter “0”.

If the project is a single-family or multi-family building 3 stories or less and is participating in the SMUD Neighborhood SolarShares program, input the PV system sized for community solar as specified in the Certificate of Compliance and select the checkbox for SMUD SolarShares and ensure SMUD is selected as the electric utility. An additional drop-down menu will also be available to select buy-down options, if applicable. For more information on the Neighborhood SolarShares program, please visit the SMUD website.

### Step 1.5: Compliance

Figure 33

Percentage % Better Than Title 24 Used for Permit: Enter the percentage above the minimum Energy Code compliance from the Certificate of Compliance used to apply for the building permit.  All projects must comply with the California Energy Code to qualify for incentives.

Figure 34

Rental Credit Provision Checkbox (optional): Check this box if the project will follow the rental credit provision per the BUILD Guidelines (2nd Edition, Chapter 5 Section A.4.a.iv). This provision is only for projects requiring a Custom Path Tool tenant PV allocation and located in a utility service territory without a VNEM or equivalent program.

## Step 1a (optional): Include any kickers to qualify for additional incentives

This step is optional and is for any technology that qualifies for kicker incentives. For more information on the kicker incentives and minimum requirements, refer to the BUILD Guidelines.

Figure 35

No. of Smart Thermostats: Enter the number of smart thermostats.

No. of JA-13 Compliant HPWHs: Enter the number of Joint Appendix 13 compliant unitary heat pump water heaters.

Lbs of Low GWP refrigerant (<150): Enter the pounds of refrigerant with less than 150 global warming potential.

Lbs of Lower GWP refrigerant (<750): Enter the pounds of refrigerant between 150 and 750 global warming potential.

No. of Electric Induction Cooktops: Enter the number of electric induction cooktops. Count only permanent fixtures, including slide in ranges.

No. of Heat Pump Clothes Dryers: Enter the number of heat pump clothes dryers.

Size of Battery System: Enter the size of the battery system in kWh. This value does not need to match the value in Step 1.5.

No. of EVSE: Enter the number of electric vehicle supply equipment.

No. of Smart EVSE: Enter the number of smart electric vehicle supply equipment.

No. of Bi-Directional EVSE: Enter the number of bi-directional electric vehicle supply equipment.

# 5. Calculations

Review calculations to ensure the modeled resident utility cost savings requirement is met.

## Step 2: Press the Calculate Button

When ready, click the CALCULATE button on the right side of the sheet in Step 2.

Figure 36

A pop-up will appear asking for the folder location containing the two or four files.

Figure 37

If there were no errors, a calculation complete message should appear.

Figure 38

## Step 3: Confirm project meets the program’s modeled resident cost savings requirement

Projects meeting the modeled resident utility cost savings requirement will see a green “YES” box. The applicant may move onto Step 5.

Figure 39

Projects not meeting the modeled resident utility cost savings requirement will see a red “NO” box.

Figure 40

## Step 4: Options to meet modeled resident utility cost savings

If the project does not meet requirements, an applicant has the option to incorporate additional building energy efficiency enhancements and/or install additional PV for tenants. Below are the three options an applicant can take to meet the modeled resident utility cost savings requirement:

- Increasing energy efficiency measures.

An applicant may go back to the model and increase the energy efficiency of the mechanical and/or envelope measures. Be sure to press the CALCULATE button in Step 2, to recalculate the results.

- Installing or allocating PV to tenants.

An applicant can agree to install the minimum PV system size needed to meet requirements (boxed in the Tool).

Figure 41

- Combination

An applicant may do a combination of efficiency measures or installing/allocating PV to tenants to meet requirements. As the energy efficiency measures increase, the minimum PV system size needed to meet requirements decrease.

Figure 42

(For Multifamily) Minimum PV system size allocated to tenants to meet requirements:

For multifamily, this is the minimum amount of PV needed to be allocated to tenants to meet the program modeled resident utility cost savings requirement.

OR

(For Single-Family) Minimum PV System size needed to meet requirements:

For single-family, this is the minimum amount of PV needed to be installed to meet the program modeled resident utility cost savings requirement.

In cases where PV is not required by the Energy Code, the italicized PV information displayed below will not apply and all PV required by BUILD is eligible for the incremental PV incentive as described in the BUILD Guidelines.

Estimated PV from code requirement (if applicable): The estimated amount of PV required for the project is based on the 2019 Energy Code. This estimate is for informative purposes only and is not intended to replace the required amount of PV under the Energy Code for the actual building.

Estimated PV eligible for the incremental PV incentive (if applicable): The amount of PV that is eligible for the incremental PV incentive. To qualify for this incentive, the PV must be used to meet the modeled resident utility cost savings requirement and only the additional PV above code is eligible.

Figure 43

Select “Yes” if agreeing to install the total PV System Size (boxed above): This section (Figure 43) appears when the modeled resident utility cost savings requirement is not met. The “Yes” must be selected to agree to installing or allocating the minimum PV system size needed to meet requirements (boxed in the Tool) and/or increase the building energy efficiency measures in Step 1 of the Tool.

Figure 44

## Step 5: Cost Savings Difference

BUILD has a requirement that the monthly modeled resident utility cost for an all-electric building shall be less than the monthly modeled resident utility cost for a similar mixed-fuel building by at least 5%. The all-electric monthly modeled resident utility cost is calculated by the Tool by applying the hourly rates from each utility’s tariff schedule to each hour in the hourly results files and calculated on a monthly basis. The same is done for the similar mixed-fuel building for the hourly electricity and gas use. The entire building’s annual utility cost is summed, divided by 12 months, and evenly divided by number of dwelling units.

Figure 45

Monthly Modeled Resident Utility Cost Difference: Estimated modeled cost difference between the monthly utility cost of a 2019 Energy Code prescriptive standard mixed fuel building and the monthly utility cost of the all-electric building per tenant bill. A positive dollar amount is cost savings, and a negative dollar amount is a cost increase. The example above (Figure 44) is a negative $1.14 prior to incremental PV, therefore the utility bill for the all-electric model is $1.14 more expensive than the mixed fuel.

Modeled Utility Cost Savings: Estimated modeled percent difference between the monthly utility cost of a 2019 Energy Code prescriptive standard mixed fueled building and the monthly utility cost of the all-electric building per tenant bill. A positive percentage is cost savings; a negative percentage is a cost increase. The example above is a negative 5%, therefore the utility bill for the all-electric bill is 5% more expensive than the mixed fuel. It needs to be positive 5% to meet the cost savings requirements of the BUILD Guidelines.

# 6. Review and Save Results

An applicant should review and save the results for use during the application process.

## Step 6: Review Results and Estimated Incentive Amount

The final calculated results and incentive levels are now displayed. The estimated avoided GHG of the building is calculated by applying the hourly GHG emission rates, per each Climate Zone, to each hour in the hourly results files and summed for the annual savings. The incentive levels are then applied to each of the incentive components.

### Step 6.1: Percent Better Than Title 24 and GHG Results

Figure 46

Estimated % Better than Title 24: This value is pulled from Step 1. Percentage above the minimum energy code compliance for increased building envelope and efficiency measures. ALL projects must comply with the California Energy Code to qualify for incentives.

Estimated Avoided GHG of Building: The estimated avoided GHG is the difference between the total annual GHG emissions of the mixed-fuel baseline building and the all-electric building.

### Step 6.2: Incentive Calculation

Figure 47

GHG Base Incentive of 30 years: The base incentive is calculated from the building’s avoided GHG emissions. The incentive amount is calculated by multiplying the total avoided GHG emissions, in metric tons, by $150 and then multiplied by 30 which represents the 30-year effective life of the building.

Increased Building Efficiency Incentive: Based on the performance of the building when compared to the Title 24 Part 6 standard design, known as the compliance margin. This is calculated by approved compliance software and can be found on the CF1R-PRF-01E or NRCC-PRF-01-E shown as a percentage.

Incremental PV Incentive: This incentive is calculated by multiplying the amount of additional PV, in Watts DC, by $1.30 for buildings where PV is already required by the Energy Code. For buildings where PV is not required by the Energy Code, such as multifamily buildings four-stories and higher, BUILD will incent the entire amount of the required PV. This is calculated by multiplying the amount of PV by $3.00 per Watt DC.

Total Kicker Incentive: This incentive is calculated by summing all the kicker incentives from Step 1a.

Total BUILD Incentive: The total BUILD incentive the total of all the incentives. There are two totals, one for the entire building and one on a per bedroom basis.

### Step 6.3: Save a Copy

Figure 48

An applicant should print a PDF of the calculator for future reference and to attach and upload to the BUILD incentive reservation application. Use the print button and follow the prompts to save.

The applicant should also save the results as a smaller .XLSX file (a file without the macro and underlying formulas), for future reference and to attach and upload to the BUILD incentive reservation application. Use the Save as XLSX button and follow the prompts to save.